# Danièle Héon – P. Geo.

12 Marigold Place

Whitehorse, Yukon

Canada          Y1A 6A2

### Ph: (867) 668-6242

# Cell: (867) 333-0510

# danieleheon@gmail.com

# www.danieleheon.com

Dempster Vanadium Project

Relogging of FX07-02 and FX07-03, December 2018

General notes

Core logging of holes FX07-02 and -03 took place at the YGS core library between December 6 and December 12, 2018, with the core sampling taking place between December 6 and 18. Samples were picked up by Bureau Veritas on the morning of December 19, 2018, with the instruction to hold until instructed before processing the samples.

The original drilling targeted a nickel sulphide horizon that is outcrops further south at the contact between the Canol and Road River formations. In the property area, this contact is recessive so therefore doesn’t outcrop.

Neither hole intersected significant mineralization. The previous operator took very few samples for assays. In hole FX07-02, the prospective contact had been logged across a zone of brecciation and quartz-carbonate veining. The maximum nickel value was 519 ppm. In hole FX07-03, the previous operator placed the prospective contact at a zone of increased abundance of carbonate pods but this author suggests that the contact be placed a bit up-section, where changes in sedimentary features, such as the appearance of these carbonate pods or bands, and the onset of pyrite sedimentation, suggest a change in sedimentation regime.  The highest nickel value in the former sampling was of 293 ppm.

Some lithological contacts were previously marked in the box on coloured papers; labelled as Litho 1,2 3, etc., these were cross-referenced or used to delimit sample intervals. Both holes are strongly carbonaceous and variable calcareous.

Sampling

The depth measurements were based on markings on blocks that mark the end of a run; these blocks usually occurred regularly at every 3 m.  Measurements are to be considered approximate due to the degradation of the core, that the core may have moved around in the boxes, and the uncertainty of how much core is missing due to previous sampling a variety of purposes.

Samples length measured generally one metre, except where core recovery was very poor, or where lithological contrast dictated the need for a different sample length. Where previously assayed, the new sample followed the same intervals as the older samples.

The core was difficult to sample due to the high degree of fracturing and degradation. The core varied from powdery to flaky to crumbly to relatively competent. In places the rock looked competent but crumbled easily along microfractures under the least pressure. Every effort was made to leave half the core in the box. Where the available core consisted of half-core, half of this was sampled, resulting in a sample consisting of a quarter core, and leaving a quarter core in the box. Most of the core was sawn, with some section split or simply scooped up.

Sample tags were stapled in the core boxes at the beginning of each sample.

QAQC

A spreadsheet with the location/frequency of QAQC samples was provided by Breakaway. The target was to have one blank, one standard, and one duplicate for every 33 regular samples.

The blank material consisted of coarsely crushed granite purchased from the BV laboratory, this is the same material that the lab uses as blanks.

A commercial reference material or standard, Oreas 520, was inserted in the sample stream.

Duplicate samples consisted of half of what remained of the core after regular sampling; this resulted in a quarter-core duplicate. The location of duplicate samples was sometimes modified from what was listed in the spreadsheet in order to correspond to core competent enough to be cut twice.

FX07-02

- Very poor recovery at top of hole.

- Some 4-5 cm section sampled/missing, also some sampling was done taking a few pieces throughout the entire hole, not obvious where these were taken from.

- Canol/Road River contact marked by brecciation and veining.

- Bottom of hole is strongly graphitic.

FX07-03

- Some carbonate replacement pods (?): narrow interval of more abundant pods is chosen by previous operator to represent top of Road River Fm.

- Canol/Road River contact suggested to be further up in hole, at appearance of fine-grained pyritic beds.

- Rocks after carbonate pod horizon look the same as the ones before.

- Some evidence of fossils, some replaced by carbonate (nautiloid?), other: graptolites.

- Presence of carbonate crystals with distinctive twinned crystals and swallowtail terminations: witherite? Calcite pseudomorphing gypsum?

- Doesn’t have the high-grade graphite of hole -02.